Name : Muhammad Abdullah

Reg # FA24-BSE-042

Instructor : Raheel Ahmed

Lab Report

Lab 12: Verify the Working of JK Flip-Flop (FF) & RS Flip-Flop (FF)

Digital Logic Design

## 2.2 The RS Flip-Flop (NOR Gate Implementation)

The SR (or RS) flip-flop is the simplest bistable circuit. It can be constructed using two cross-coupled NOR gates or two cross-coupled NAND gates. The NOR-based RS flip-flop has active-HIGH inputs (S and R), while the NAND-based version uses active-LOW inputs (S̄ and R̄).

NOR-Gate RS FF Truth Table (Active-HIGH):

| S | R | Q (Next State) | Q̄ (Next State) | Operation |
| --- | --- | --- | --- | --- |
| 0 | 0 | Q (No change) | Q̄ (No change) | Hold / Memory |
| 0 | 1 | 0 | 1 | Reset |
| 1 | 0 | 1 | 0 | Set |
| 1 | 1 | Forbidden | Forbidden | Invalid / Undefined |

Key observations for the NOR-gate RS FF:

- S=0, R=0: The circuit holds its previous state (memory state).

- S=0, R=1: The flip-flop is RESET — Q goes to 0.

- S=1, R=0: The flip-flop is SET — Q goes to 1.

- S=1, R=1: Both outputs attempt to go to 0 simultaneously, creating a forbidden/undefined state that must be avoided.

## 2.3 Circuit Simulation — RS Flip-Flop (NOR Gates)

The following Proteus simulation shows the NOR-gate based RS flip-flop with S=1, R=1. The outputs P (≡Q) and Q are monitored via LED indicators.

Figure 1: RS Flip-Flop using NOR gates (Proteus Simulation) — S=1, R=1

Simulation Analysis:

- In the simulation, S is set HIGH (1) and R is also HIGH (1).

- Both LEDs D1 and D2 are OFF, indicating both P (Q) and Q outputs are LOW — this is the forbidden state.

- In practice, this condition must be avoided as it leads to unpredictable behavior when inputs return to 0,0.

- The circuit correctly demonstrates the cross-coupled NOR gate behavior.

## 2.4 The Clocked RS Flip-Flop (Synchronous)

In digital systems, it is often necessary to synchronize flip-flop state changes with a clock signal. The clocked RS flip-flop adds two NAND gates at the input to gate the S and R signals with the clock. The flip-flop only responds to S and R when the clock (CLK) is HIGH.

Using NAND gates at the front end makes S and R active-HIGH in this configuration. The truth table below describes behavior on the nth clock pulse:

Clocked RS Flip-Flop Truth Table:

| CLK | Rₙ | Sₙ | Qₙ₊₁ | Q̄ₙ₊₁ | Operation |
| --- | --- | --- | --- | --- | --- |
| 0 | X | X | Qₙ | Q̄ₙ | No effect (clock disabled) |
| 1 | 0 | 0 | Qₙ | Q̄ₙ | No change (hold) |
| 1 | 0 | 1 | 1 | 0 | Set |
| 1 | 1 | 0 | 0 | 1 | Reset |
| 1 | 1 | 1 | Undefined | Undefined | Forbidden |

## 2.5 The JK Flip-Flop

The JK flip-flop is an improvement over the RS flip-flop. It eliminates the forbidden state by converting it into a useful toggle state. The inputs are renamed J (analogous to S/Set) and K (analogous to R/Reset).

The critical improvement: when both J=1 and K=1, instead of entering an undefined state, the flip-flop toggles (inverts) its current output. This makes the JK flip-flop the most versatile and widely-used flip-flop type.

JK Flip-Flop Truth Table:

| CLK | Jₙ | Kₙ | Qₙ₊₁ | Q̄ₙ₊₁ | Operation |
| --- | --- | --- | --- | --- | --- |
| ↑ | 0 | 0 | Qₙ | Q̄ₙ | No change (hold) |
| ↑ | 0 | 1 | 0 | 1 | Reset |
| ↑ | 1 | 0 | 1 | 0 | Set |
| ↑ | 1 | 1 | Q̄ₙ | Qₙ | Toggle |

JK Flip-Flop Operation Summary:

- J=0, K=0: Output remains unchanged — hold state.

- J=0, K=1: Output is RESET to Q=0.

- J=1, K=0: Output is SET to Q=1.

- J=1, K=1: Output TOGGLES — Q becomes Q̄ (the complement of the previous state).

The toggle feature is especially useful in binary counters where the output must alternate on every clock pulse.

## 2.6 JK FF Implementation Using AND + NOR Gates

The JK flip-flop can be implemented by combining AND gates at the inputs (to incorporate J, K, CLK, and feedback from Q/Q̄) with a cross-coupled NOR gate SR latch at the output. This is the circuit shown in the simulation below.

- AND gate U1 computes: S = J · CLK · Q̄ (set condition)

- AND gate U2 computes: R = K · CLK · Q (reset condition)

- NOR gates U3 and U4 form the SR latch holding the state.

- Feedback from Q and Q̄ back to U1/U2 ensures toggling when J=K=1.

## 2.7 Circuit Simulation — JK Flip-Flop (AND + NOR Gates)

Figure 2: JK Flip-Flop using AND and NOR gates (Proteus Simulation) — J=0, K=0, CLK=1

Simulation Analysis (J=0, K=0, CLK=1):

- With J=0 and K=0, both AND gate outputs (S and R) remain LOW regardless of CLK.

- The NOR latch holds its previous state — Q=0 and Q'=0 shown on the output probes.

- This verifies the 'No Change' row of the JK truth table.

- The clock is active (CLK=1), but since J=K=0, no state change occurs.

## 2.8 Clocked SR Flip-Flop Using NAND Gates

The clocked SR flip-flop can also be implemented using NAND gates. In this NAND-based configuration:

- NAND gates U1 and U2 serve as the input gating stage, combining S (or R) with the CLK signal.

- NAND gates U3 and U4 form the cross-coupled SR latch.

- Since NAND gates are used throughout, the latch portion has active-LOW behavior, but the overall circuit with the gating stage produces active-HIGH S and R behavior.

- The inputs are only effective when CLK=1 (HIGH).

## 2.9 Circuit Simulation — Clocked SR FF (NAND Gates)

Figure 3: Clocked SR Flip-Flop using NAND gates (Proteus Simulation) — S=1, CLK=1, R=0

Simulation Analysis (S=1, CLK=1, R=0):

- S=1, CLK=1: The NAND gate U1 output goes LOW (since both inputs are HIGH), driving the SET condition into the latch.

- R=0: U2 output remains HIGH, keeping the RESET input inactive.

- Result: Q=1 (output indicator shows 1) and Q'=0 — the flip-flop is SET.

- This verifies the SET row of the clocked SR flip-flop truth table when CLK=1, S=1, R=0.

# 3. Equipment & Procedure

## 3.1 Equipment Used

- Computer with Proteus Design Suite (simulation software)

- Logic gates: NOR (74HCT02), NAND (74HCT00), AND gates

- Logic switches/probes for input/output monitoring

- LED indicators for output visualization

## 3.2 Procedure

Part A — NOR-gate RS Flip-Flop:

- Connect two NOR gates in a cross-coupled configuration (output of each feeds into input of the other).

- Connect S and R inputs to logic switches; Q and Q̄ to LED probes.

- Test all four input combinations: (0,0), (0,1), (1,0), (1,1) and record outputs.

- Observe memory behavior when both S and R return to 0 after being 0,1 or 1,0.

Part B — JK Flip-Flop (AND + NOR):

- Connect AND gates U1 (J, CLK, Q̄) and U2 (K, CLK, Q) as the input stage.

- Feed U1/U2 outputs into a cross-coupled NOR latch (U3, U4).

- Connect feedback: Q to U2's third input, Q̄ to U1's third input.

- Test all combinations of J, K with CLK pulse and verify the truth table.

Part C — Clocked SR FF (NAND Gates):

- Wire NAND gates U1 (S, CLK) and U2 (R, CLK) as the input stage.

- Connect their outputs into the cross-coupled NAND latch (U3, U4).

- Set CLK=0 and verify inputs have no effect; set CLK=1 and test S,R combinations.

# 4. Results & Observations

## 4.1 RS Flip-Flop (NOR Gates) — Verified Truth Table

| S | R | Q | Q̄ | Operation | Observation |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | Qₙ (hold) | Q̄ₙ (hold) | Memory | Output unchanged ✓ |
| 0 | 1 | 0 | 1 | Reset | Q goes LOW ✓ |
| 1 | 0 | 1 | 0 | Set | Q goes HIGH ✓ |
| 1 | 1 | X | X | Forbidden | Undefined state ✓ |

## 4.2 JK Flip-Flop — Verified Truth Table

| CLK Edge | J | K | Q (next) | Q̄ (next) | Operation | Verified |
| --- | --- | --- | --- | --- | --- | --- |
| ↑ | 0 | 0 | Qₙ | Q̄ₙ | No Change | ✓ |
| ↑ | 0 | 1 | 0 | 1 | Reset | ✓ |
| ↑ | 1 | 0 | 1 | 0 | Set | ✓ |
| ↑ | 1 | 1 | Q̄ₙ | Qₙ | Toggle | ✓ |

## 4.3 Clocked SR FF (NAND) — Verified Truth Table

| CLK | S | R | Q (next) | Q̄ (next) | Operation |
| --- | --- | --- | --- | --- | --- |
| 0 | X | X | Qₙ | Q̄ₙ | Clock disabled — no change |
| 1 | 0 | 0 | Qₙ | Q̄ₙ | Hold |
| 1 | 0 | 1 | 0 | 1 | Reset |
| 1 | 1 | 0 | 1 | 0 | Set |
| 1 | 1 | 1 | Undefined | Undefined | Forbidden |

# 5. Conclusion

This lab successfully demonstrated and verified the operation of RS and JK flip-flops through Proteus simulations:

- The NOR-gate RS flip-flop correctly exhibited Set, Reset, Hold, and Forbidden states as predicted by theory. The memory characteristic  retaining the last valid state when S=R=0 — was clearly observed.

- The clocked SR flip-flop added synchronization control, ensuring state changes occur only during active clock pulses, which is fundamental to synchronous digital design.

- The JK flip-flop resolved the forbidden-state problem of the RS FF by introducing the toggle functionality when J=K=1. This makes the JK FF the most versatile flip-flop and the preferred choice in counters and state machines.

- The simulation results matched the expected truth tables for all three circuits, confirming the correctness of the designs.

- Feedback connections from the Q and Q̄ outputs back to the input AND gates in the JK FF are critical for implementing the toggle behavior.

Overall, flip-flops form the backbone of sequential logic. Their ability to store a single bit of information and change state in a controlled manner under clock and input signals is the foundation of all digital memory, counters, shift registers, and state machines.