LAB REPORT # 07

Submitted by:

Submitted by:

| Name | Registration No. |
| --- | --- |
| Muhammad Abdullah | FA24-BSE-042 |

Submitted to: Raheel Ahmed

Course: Fundamentals of Digital Logic Design(EEE240)

Dated: MAR 29, 2026.

DEPARTMENT OF COMPUTER SCIENCE

COMSATS UNIVERSITY ISLAMABAD,ISLAMABAD CAMPUS

# 4-Bit Adder :

## Circuit:

## Truth Table:

| A | B | Cin | Sum (S) | Carry (Cout) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

# 4-Bit Subtractor :

## Truth table:

| A | B | B' (B⊕1) | Cin | Difference (D) | Cout | Borrow |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 | 1 | 0 |
| 1 | 1 | 0 | 1 | 0 | 1 | 0 |

# Conclusion

In this lab, we learned how binary addition and subtraction can be implemented using digital logic circuits. We first studied the operation of a 1-bit full adder, which forms the foundation for building multi-bit arithmetic circuits. By cascading four full adders, we designed a 4-bit ripple-carry adder capable of performing binary addition.

We also learned how the same adder circuit can perform subtraction through the 2’s complement method, where the subtrahend (B) is inverted and a logic 1 is added as Cin. This allowed us to perform A – B using the same hardware used for addition. The lab demonstrated the role of logic gates such as XOR, AND, and OR in generating sum and carry/borrow bits.

Overall, this experiment helped us understand:

- The internal working of a full adder

- Cascading full adders to build multi-bit arithmetic units

- How subtraction is achieved using 2’s complement

- The practical importance of ripple-carry propagation

- Implementing and verifying results using simulation